MXEN2003 – Micro Controller Project

Laboratory Logbook

Lab 1

Aaron Fredrick (20932390) 7-3-2024

Contents

Task 1	1

Objectives	1

Calculations	1

Explanations	1

Task 1.1	2

Equipment Used	2

Circuit Diagram	2

Program Logic	3

Code Snippet	3

Procedure & Methodology	4

Task 1.2	5

Equipment Used	5

Circuit Diagram	5

Program Logic	6

Code Snippet	7

Procedure & Methodology	7

Task 1.3	8

Equipment Used	8

Circuit Diagram	8

Program Logic	9

Code Snippet	10

Procedure & Methodology	10

Reflection	11

Appendix	12

Part 1: Images of physical circuit of Task 1.2	12

# Task 1

## Objectives

The objective of this task is to program an Arduino Mega 2560 to control a series of 8 LEDs connected to PORTA. The procedure involves breaking down the task into subtasks to understand the fundamentals of LED control. Here are the subtasks:

- Task1.1: Alternatively blinking 2 LEDs on PORTA.

- Task1.2: Sequentially blinking 8 LEDs on PORTA.

- Task1.3: Sequentially blinking 8 LEDs on a different port than PORTA.

## Calculations

With reference to the provided data sheet, the maximum current through a (red) LED is 20mA and the voltage output from a digital pin is 5V.

R=VI

R=50.02A=250Ω

## Explanations

Pins on the Arduino Mega are often identified by their numerical values, which match the pin numbers on the board. Furthermore, the digital pins are arranged into distinct regions designated as ports A to K. As each port has eight pins and being able to adjust with bitwise operations, it allows for better and effective control of several pins at once.

When doing bitwise operations on these pins, the symbols | (bitwise OR) and & (bitwise AND) are essential. They make it possible to compare and work with individual bits within of a byte. These symbols make it easier to do actions like simultaneously setting several pins to HIGH or LOW by comparing each positional bit with its equivalent bit in another byte.

Removing these symbols would alter the behavior of pin manipulation. Without bitwise operations, updating the port with a new byte would involve individually setting or clearing each pin.

## Task 1.1

### Equipment Used

- Arduino Mega 2560 Board

- Bread Board

- 2 x 330Ω Resistors

- 2 x LEDs

- Jumper Wires

### Circuit Diagram

Figure 1- Lab1 task 1.1 circuit diagram

### Program Logic

Figure 2- Lab1 task 1.1 flow chart

### Code Snippet

### Procedure & Methodology

- Initially we connect the 2 LEDs to resistor each individually in series.

- Then the circuit is completed by connecting the cathode of the bulbs to ground and the anode sides(resistors) to pin 22 and 23.

- Next, we programmed the Arduino board to set pin 22 and 23 as output mode and send HIGH and LOW digital outputs alternatively.

  - Since we are using atmelavr platform we cannot use the inbuild Arduino libraries instead we use AVR libraries.

  - In AVR pin 22 and 23 belongs to A region (Port A) and since we need to send output signals we set Data Direction Register of A (DDRA) to 0xFF which is equivalent highest 8-bit byte value in hex (looking at the 8-bit representation, 0b11111111 it is not necessary to set all bits as high since we are only using the first 2 pins or in other words the first 2 LSBs of this byte).

  - Following the DDR configuration, we use the PORTA defined variable to set the required high and low values, and in 8-bit representation at one instance it should be 0b00000001 and in alternative state it should be 0b00000010, hence this could be programmed as follows:

Or in bit logics and bitwise operations:

But since this a simple program we can reduce the code rows and columns by representing 0b00000001 as 1 and 0b00000010 as 2 and program it as shown here

## Task 1.2

### Equipment Used

- Arduino Mega 2560 Board

- Bread Board

- 8 x 330Ω Resistors

- 8 x LEDs

- Jumper Wires

### Circuit Diagram

Figure 3- Lab1 task 1.2 circuit diagram

### Program Logic

Figure 4- Lab1 task 1.2 flow chart

### Code Snippet

### Procedure & Methodology

- Initially we connect the 2 LEDs to resistor each individually in series.

- Next, complete the circuit by connecting the cathodes of the bulbs to ground and the anode sides (resistors) to pins 22 to 29.

- Finally, we programmed the Arduino board to set pin 22 to 29 as output mode and send HIGH and LOW digital outputs sequentially.

  - Since we are using atmelavr platform we cannot use the inbuild Arduino libraries instead we use AVR libraries.

  - In AVR pins 22 to 23 belongs to A region (Port A) and since we need to send output signals we set Data Direction Register of A (DDRA) to 0xFF which is equivalent highest 8-bit byte value in hex.

  - Following the DDR configuration, we use the PORTA defined variable to set the required high and low values, in 8-bit representation at one instance it should go from 0b00000001 to 0b100000000, and this shows a pattern where it shift the bits to left hence we add a for loop which shifts a value of 1 in 8-bit to left for a given iteration index at a specific iteration which in end satisfies the sequential blinking of the 8 LEDs.

## Task 1.3

### Equipment Used

- Arduino Mega 2560 Board

- Bread Board

- 8 x 330Ω Resistors

- 8 x LEDs

- Jumper Wires

### Circuit Diagram

Figure 5- Lab1 task 1.3 circuit diagram

### Program Logic

Figure 6- Lab1 task 1.3 flow chart

### Code Snippet

### Procedure & Methodology

- Similar to Task 1.2 the circuit is setup as it is, except for the digital connections, we use PORTC which are the pins 30 to 37.

- Finally, the program is coded to sequentially blink 8 LEDS connect to PORT C.

  - In AVR pins 30 to 37 belongs to C region (Port C) and since we need to send output signals, we set Data Direction Register of C (DDRC) to 0xFF which is equivalent highest 8-bit byte value in hex.

  - Following the DDR configuration, we use the PORTC defined variable to set the required high and low values, and the blinking logic will be the same as Task 1.2 except the set variable used, will be PORTC.

# Reflection

In this lab, we learned how to effectively utilize AVR libraries and manipulate PORT regions to control inputs and outputs on an AVR microcontroller. One of the main things we grasped was configuring DDR settings to define whether pins act as inputs or outputs.

By making use of AVR libraries, we simplified our code and gained access to useful functions, boosting our efficiency. This allowed us to achieve our project goals more smoothly and efficiently.

Additionally, we honed our skills in bit manipulation and logical operations, which helped us control outputs in specific patterns as needed for our project. This hands-on experience enhanced our understanding of low-level hardware interactions.

Overall, this lab provided valuable insights into AVR programming, equipping us with practical skills and confidence to tackle future projects with AVR microcontrollers.

# Appendix

## Part 1: Images of physical circuit of Task 1.2